Magnet Math	Names ______________________________________________________________________________ Period ____

1. A transformer has 50 turns on the primary and 10 turns on the secondary.  There are 1000 V and  2A on the primary.

(a) What is the power in watts on the primary?

(b) What is the power on the secondary?

(c) What is the voltage on the secondary?

(d) What are the amps on the secondary?

2. A transformer has 500 turns in the primary and 3000 turns in the secondary.  In the primary, the potential difference is 120 volts AC and the current s 150 milliamperes (0.150 A).  Find

(a) What is the power on the primary side of the transformer?

(b) The ratio of primary to secondary.  Is this a step-up or step-down transformer?

(c) What is the voltage on the secondary?

(d) What is the amperage on the secondary?

3.  A transformer on the primary side has 4000 turns, 120 volts and 10 amps.

(a) How many turns are needed in the secondary to step-down the voltage to a 9-V charger?

(b) What is the amperage going to your cell phone?

4.  Transformers are notorious for overheating and losing energy.  A transformer has 2400 V, 6 A on the primary side and120 volts, 100 A on the secondary side.

(a) What is the efficiency of the transformer?

(b) Using the information for the voltage, if there are 100 turns on the secondary side, how many are on the primary?

(c) Using the information for the amperage, if there are 100 turns on the secondary side, how many are on the primary?

4.  A transformer is 90% efficient meaning some energy is lost to heat.  The transformer has a ratio of 100:1 and has 12,000 volts, 1 A while the secondary has 120 volts.

(a) What is the power in the primary?

(b) What should the power be in the secondary?

(c) What should be the amperage in the secondary if it were 100% efficient?

(d) What is the actually power in the secondary?

(e) What is the actual amperage in the secondary?

| Location | Primary : Secondary Ratio | Voltage | Amperage |
| --- | --- | --- | --- |
| Plant |  | 6000 |  |
| High Voltage Lines in City |  | 120,000 |  |
| Ultra High Voltage (Cross Country) |  | 480,000 |  |
| High Voltage Lines in City |  | 120,000 |  |
| Low Voltage Wires in neighborhood |  | 2400 |  |
| Local and Homes | 1 | 120 | 100 |